Documentation of the code developed for the Ultrasonic Ranger Module

Introduction:

The code waits for a character to be received serially on its. Upon receiving any character, it initiates the process of estimating the distance to the closest obstacle in front of it.

Sending the 40KhZ Burst:

As soon as a character is received, a 40KhZ pulse train (approximately 10 pulses) is sent out of the receiver. The number of pulses that are sent out can be increased or decreased by changing the constant parameter NUMBER_OF_PULSES. In our experimentation we found that for longer range distance estimation, a larger number of pulses enabled us to get more accurate readings.

Waiting for cross talk to subside:

If the receiver is switched on at the time of sending the burst, the receiver is likely to pick up that signal and hence the system may gave an erroneous reading of the distance. Therefore the systems waits for a certain period of time before activating the interrupts and the receiving circuit. This waiting is done at a frequency given by fCLK/1024 where fCLK is the system clock frequency. The amount of waiting time can be controlled by CROSSTALK_WAIT parameter.

Distance Estimation:

The interrupt is triggered when the 40KhZ burst, after reflection from an obstacle is received. During this time the timer, TIMER0 may overflow a number of times. The number of times the timer overflows is recorded in a variable time_major. When the external interrupt is triggered, the timer interrupt is disabled and the number of times the timer overflowed (time_major) and the current readout of the timer counter, TCNT0 (time_minor) are recorded. This timer clock is driven by a frequency = fCLK/64 . If the timer overflows for forty times without the external interrupt having been triggered, the processor stops waiting for the external interrupt and returns with the value of 40 of time_major indicating that no 40KhZ train was received. It would be important to note that given the maximum range of this ranging module, time_major=40 would not be an achievable value under ranging. Therefore the value 40 indicates that the burst has not been received and may be interpreted as a "time out".

Communication - SOFT UART :

The AtTiny45 microcontroller doesn't have a UART implemented. A soft UART, that is UART in software has been implemented. The separate header file "uart.h" contains the functions for receiving and transmitting the data serially. The main function uses send_char() and recv_char() functions to send and receive characters serially. Currently the settings of serial transmission are 9600bps, no parity bit and 1 stop bit. The implemented UART enables the ranging module to be connected to another microcontroller and communicate serially or be connected to the computer via an external level shifter circuit and communicate serially.

Distance Calculations based on the data returned:

Let us assume the system clock frequency to be fCLK Hz.The timer at the time of waiting for cross talk to subside is driven by the system clock and by a frequency of f1 =

fCLK/1024. The timer at the time of waiting for the pulse  is driven by the system clock is driven by a clock frequency of f2 = fCLK/64.

Let T1=1/f1 and T2=1/f2

The total time that it takes for the burst to go and return is given by -

( (time_major*255 ) * T2 ) + (time_minor * T2) + (CROSSTALK_WAIT * T1)

This gives the total travel time from the transmitter to the obstacle and back. The distance can be estimated as one half of the value obtained by multiplying this time by the speed of sound in air.

Longer Range vs Shorter Range, Minimum Resolution:

Different transmitting modules have been employed to cater to longer ranges of distance estimation. For shorter ranges, the intensity of the transmitted burst (power transmitted) and the number of pulses is usually less, therefore the cross talk wait time can be kept to a lower level. In the case of larger values of transmitted power, we would need to keep the cross talk wait time to a larger value.

The change in cross talk wait time affects the minimum range of the ranging module. This is because, when the module is waiting for cross talk to subside, it would not be able to register the 40KhZ burst that may have reflected from a nearby object.

The minimum resolution can be calculated as -

d1 = ( CROSSTALK_WAIT * T1 ) * (speed of sound in air)

d=d1/2 gives us the minimum distance of an obstacle from the ranger module.

Incorporating distance calculation in the microcontroller itself

The microcontroller code can be modified to return distance in required units rather than returning the value of time_major and time_minor.